Experiment 10

SPEED OF SOUND

APPARATUS:	Ultrasonic crystals, audio generator, oscilloscope

REFFERENCE:	Rossing, The Science of Sound, Section 3.5

DISCUSSION:

The goal of this experiment is to experimentally determine the speed of sound in air using ultrasonic transducers. One transducer will function as a sound wave transmitter; the other transducer will function as a sound wave receiver.

PROCEDURE:

1.	Drive the transmitting crystal with an audio generator and connect the receiving crystal to the vertical input of an oscilloscope. Spend a few minutes familiarizing yourself with the qualitative behavior of the transducers.

Determine the resonant frequency of the crystals.

2.	Devise a method of measuring the wavelength of the sound wave emitted by the transmitting transducer. It may be useful to connect the receiving crystal to the vertical input of the oscilloscope, and the generator output to the horizontal input, to display a fy vs fx Lissajous pattern whose shape varies with the phase of the signal from the receiver.

Be sure to be able to discuss the precision with which you can determine the wavelength.

3.	Record the frequency of the signal from the audio generator with a frequency counter.  Calculate the speed of sound. Compare it to the value given in Table 3.1 of Rossing (corrected to the temperature in your room.)

If time permits:

4.	Measure the attenuation of 40 kHz ultrasound by various materials (paper, Styrofoam, etc.) by noting the difference in amplitude of the signal with and without the absorber in its path.

DATA SHEET AND MINI-REPORT : Experiment 10

(To be used if a laboratory notebook is not kept)

= 	m	v = f

t =	C	T =	K

v(T) =

2, 3)

|  | 10 | f |
| --- | --- | --- |
|  | cm | Hz |
| Average |  |  |

Discussion: Estimate the uncertainty in  and f. Call these  and f.

Calculate the uncertainty in speed v from the formula:

Does v  v span the calculated value v(T)?

4)