Experiment 10

SPEED OF SOUND

APPARATUS:	Ultrasonic crystals, audio generator, oscilloscope

REFFERENCE:	Rossing, The Science of Sound, Section 3.5

DISCUSSION:	The speed of sound in an ideal gas is given by:

Where T is absolute temperature, M is molecular weight, and R is the gas constant.  Note that the speed varies directly with temperature but is independent of pressure.  (In a real gas, there is a slight dependence on pressure, but it is quite small).

In this experiment, the speed of sound is determined by measuring frequency and wavelength and using the relationship:  v = f.  The sound source and receiver are both ceramic crystals of the type used in television remote control units.  They resonate at about 40 kHz, well above the limit of hearing (dogs or bats might not enjoy this experiment!).

PROCEDURE:

1.	Drive the transmitting crystal with an audio generator, and connect the receiving crystal to the vertical input of an oscilloscope.  With receiver and transmitter 5 – 10 cm apart, tune the generator to the resonant frequency of the crystals.  (you may have to retune once or twice during the experiment, but after initial warmup the audio generators are quite stable).

2.	To measure the wavelength, vary the distance between the crystals, and locate positions which are some integral number of wavelengths apart.  This can be done by connecting the receiving crystal to the vertical input of the oscilloscope, and the generator output to the horizontal input, to display a fy vs fx Lissajous pattern whose shape varies with the phase of the signal from the receiver.

Since the precision of the result depends strongly on the measurement of the wavelength, it is wise to average several measurements to determine the best value of .  one way to do this would be to make 6 or more determinations of 10 using different initial spacings of receiver and transmitter (why measure 10 rather than ?)

3.	Measure the frequency of the audio generator with a frequency counter.  Calculate the speed of sound from the formula: v = f.  Compare it to the value given in Table 3.1 of Rossing (corrected to the temperature in your room.)

If time permits:

4.	Measure the attenuation of 40 kHz ultrasound by various materials (paper, Styrofoam, etc.) by noting the difference in amplitude of the signal with and without the absorber in its path.

DATA SHEET AND MINI-REPORT : Experiment 10

(To be used if a laboratory notebook is not kept)

= 	m	v = f

t =	C	T =	K

V(T) =

2, 3)

|  | 10 | F |
| --- | --- | --- |
|  | cm | Hz |
| Ave |  |  |

Discussion: Estimate the uncertainty in  and f. Call these  and f.

Calculate the uncertainty in speed v from the formula:

Does v  v span the calculated value v(T)?

4)