Experiment 10

SPEED OF SOUND

APPARATUS:	Ultrasonic crystals, audio generator, oscilloscope

DISCUSSION:	The speed of sound in an ideal gas is given by:

Pos is position				C is speed of sound				t is time

Your ultrasonic transducers may have a different resonant frequency. You may need to check with your instructor.

The sound waves are reflecting off the surface and back causing the change in time to be double, in order to account for this delta t is divided by 2.

PROCEDURE:

1.	Drive the transmitting crystal with an audio generator and connect the receiving crystal to the vertical input of an oscilloscope. With the receiver and transmitter next to each other and a solid flat surface 5 cm in front of them, tune the generator to the resonant frequency of the crystals.

2.	Set the function generator to resonant frequency with 5V peak to peak (Vpp) and use burst at 6 cycles with one second burst period. Set the oscilloscope by using both cursors (one for beginning of the transmitter wave and one for the beginning of the receiver wave) and use time scale 1 millisecond division.

3.   Take many recordings of the position of the solid flat surface vs the time delay (the delta t). Record in the table attached.

4. 	In Logger Pro, input time delay in the x column. Then input the position in the y column. The final product will have (delta t/2) in seconds (if your time is in milliseconds, divide by 1000) on the x-axis and position in meters (if your position is in cm, divide by 100 to get to meters) on the y-axis. When this is achieved, click analyze > linear fit. The slope should be around 343 m/s.

DATA SHEET AND MINI-REPORT: Experiment 10

| Position (cm) | Delta t (ms) |
| --- | --- |
| 5 |  |
| 6 |  |
| 7 |  |
| 10 |  |
| 12 |  |
| 14 |  |
| 16 |  |
| 18 |  |
| 20 |  |
| 24 |  |
| 28 |  |
| 32 |  |
| 36 |  |
| 40 |  |
| 44 |  |
| 48 |  |
| 52 |  |
| 56 |  |
| 60 |  |

C = ___________ m/s

*The specific numbers chosen for the position do not matter. The above numbers are suggestions not recommendations.